3/23/2026 – 3/29/2026

OLED Display Visualization Ideas

Timothy Kareng

3/22/2026

Brainstorm:

- Have a countdown appear on screen for when we play along

- A start button?

- Progress bar?

- Record circle pulser display?

- Aux port to hear the song while playing?

- Use a 2.42 inch oled LCD display

3/23/2026

Adafruit 5” 800x480 HDMI Backpack (+ AR1100 Touch Controller) PWM-able Backlight

- It has micro usb port and a hdmi port

- NOT really an OLED LCD display

- Acts more like a monitor than a screen that is programmable

- The display gets pretty hot after connecting the power straight from my laptop to screen

  - Maybe need a lower voltage power source

  - Backlight is PWM-able

    - So you can lower brightness

- If we do Jetson + HDMI display

  - This is easier for the display, but heavier for the overall system.

    - a Python app with a GUI toolkit such as PyQt or Tkinter

    - or a Fullscreen local webpage served by a small app

    - or a simple OpenCV / Pygame style visualization

  - Pros:

    - Not limited to the tools used to make a OLED display work

    - Directly connected to Jetson, so communication is local

    - Bigger display with more pixels with colors (not just 1 color)

    - Easier to debug

    - Could make companion app connect easier with hardware

  - Cons:

    - Probably needs more power

    - UI can be developed faster

    - Heavier

    - Could come with OS/software issues compared to a simple microcontroller

  - Concern: How easy will it be to connect a button/switch to the jetson so that we can switch modes and other options on our device?

    - NVIDIA’s jetson-gpio repo includes event detection and PWM samples, which is a good sign for straightforward input handling.

    - library specifically to read and control those pins with an API similar to Raspberry Pi GPIO.

    - Shouldn’t be a problem then

- If we do Microcontroller + OLED/LCD display

  - Pros:

    - Less power usage

    - Smaller and cheaper

    - Can use SPI and I2C

    - Less software complexity

  - Cons:

    - Microcontroller needs communication with Jetson to work

    - Will probably have to have a smaller UI design unless we find a bigger screen

    - Less room for graphics

Start Experimenting with the Display without the Jetson

- Use your computer as a stand-in for the Jetson

- Develop a fullscreen application prototype of the Chordsense UI

- Feed it some chords to re-enact how the user will see it

- Determine what should be on the UI, what buttons should be there

- Determine what looks good in a distance and what looks clear

- Use your keyboard to model the button pressing on the device itself

- Can use Python GUI to make the display

  - https://pyuibuilder.com/

  - Can I use rust with this?

    - Yes your can use the RUST GUI: egui